🧠 PROJECT 03 — MULTI-SWITCH VLAN NETWORK WITH DHCP AND STATIC ROUTING

📌 Scenario
A growing company has expanded and now has two switches connecting different departments:

- Finance Department (VLAN 30)

- IT Department (VLAN 20)

- HR Department (VLAN 10)

- Sales Department (VLAN 40)

Management wants:

- Devices in the same VLAN on different switches to communicate.

- Automatic IP assignment using DHCP for some VLANs.

- Static routing for communication between VLANs if the router is used.

📋 Network Requirements

VLAN Information

| VLAN ID | VLAN Name | Required Hosts |
| --- | --- | --- |
| 10 | HR | 10 |
| 20 | IT | 20 |
| 30 | Finance | 30 |
| 40 | Sales | 15 |

IP Addressing

- Use 192.168.20.0/24 network.

- Apply VLSM to allocate subnets for each VLAN.

- Assign the first usable IP of each subnet to the router as the default gateway.

DHCP

- Finance VLAN (30) → Static IPs

- IT VLAN (20) → DHCP

- HR VLAN (10) → DHCP

- Sales VLAN (40) → DHCP

🧩 Devices to Use

- 1 × Cisco Router

- 2 × Cisco Switches

- 8 × End Devices (PCs / Laptops)

🔧 Tasks to Perform

1️⃣ IP Addressing

- Design the VLSM addressing scheme.

- Assign static IPs for Finance VLAN PCs.

- Configure DHCP pools for IT, HR, and Sales VLANs.

2️⃣ Switch Configuration

- Ensure VLANs 10, 20, 30, 40 exist on both switches.

- Assign access ports to the appropriate VLANs.

- Configure trunk ports between switches.

- Configure router trunk port for inter-VLAN routing.

3️⃣ Router Configuration

- Configure subinterfaces for each VLAN on the router.

- Apply 802.1Q encapsulation.

- Assign IP addresses to subinterfaces.

- Configure static routing if needed.

4️⃣ Verification

- Verify VLAN connectivity across switches.

- Verify inter-VLAN communication.

- Use ping to confirm DHCP address assignment.

- Test default gateway access for all VLANs.

✅ Expected Outcome

- Devices in the same VLAN can communicate across switches.

- DHCP works for IT, HR, and Sales VLANs.

- Finance VLAN devices retain static IPs.

- Inter-VLAN communication is functional via router.

- Router acts as the default gateway for all VLANs.

📝 Deliverables (for GitHub)

- Packet Tracer file (.pkt)

- Screenshots of:

  - VLAN creation

  - Trunk configuration

  - Router subinterfaces

  - DHCP configuration

  - Successful inter-VLAN pings

- README.md explaining the design and results

🔒 Important Rules
❌ Do NOT use a Layer-3 switch for inter-VLAN routing.
❌ Use Router-on-a-Stick.
❌ Finance VLAN should not use DHCP.